PART 1	GENERAL

1.1	SUMMARY

- Provide all equipment and materials, and do all work necessary to furnish and install the Combination Football/Soccer Goal, as indicated on the drawings and as specified herein.

1.2	RELATED WORK

- Examine Contract Documents for requirements that affect work of this section.  Other specification sections that directly relate to work of this Section include, but are not limited to:

  - Section 02200 - Earthwork; Excavation and Backfill and establishment of subgrade elevations.

  - Section 02511 - Asphalt Concrete Pavement

  - Section 02535 - Synthetic Track Surfacing

  - Section 02542 - Synthetic Field Surfacing

  - Section 03300 - Cast-in-Place Concrete; Concrete foundations and bases for goals.

1.3		REFERENCES

- Comply with applicable requirements of the following standards.  Where these standards conflict with other specified requirements, the most restrictive requirements shall govern.

      - U.S. Tennis Court and Track Builders Association

      - National Federation of State High School Associations (NFSHSA)

      - National Collegiate Athletic Association (NCAA)

      - International Amateur Athletic Foundation (I.A.A.F.)

      - Manufacturers Data and Recommended Installation Requirements.

1.4		SUBMITTALS

- Manufacturers Product Data

            - Provide manufacturer’s product literature, technical specifications and other data prior to actual field installation work for Architect or Owner’s Representative review.

# Shop Drawings

  - Provide drawings of manufacturers recommended installation and foundation requirements prior to actual field installation work for Architect or Owner’s Representative review.

1.5		QUALITY ASSURANCE

- Manufacturers warranties shall pass to the Owner and certification made that the product materials meet all applicable grade trademarks or conform to industry standards and inspection requirements.

1.6	DELIVERY, STORAGE AND HANDLING

- Materials delivered to the site shall be examined for concealed damage or defects in shipping. Any defects shall be noted and reported to the Owner’s Representative.

- Replacements, if necessary, shall be immediately re-ordered, so as to minimize any conflict with the construction schedule.

- Sound materials shall be stored above the ground under protective cover or indoors so as to provide proper protection.

PART 2	PRODUCTS

2.1		MANUFACTURERS

- Manufacturers and product selections named are provided to establish the minimum standard.

      - Combination Football/Soccer Goals - As manufactured by AAE (Aluminum Athletic Equipment Co.); 1000 Enterprise Drive, Royersford, PA 19468; Toll Free (800) 523-5471.

2.2		combination football/soccer Goals

- CAG Model - Shall be as manufactured by AAE:

  - Model # CAG-HS, Uprights: (four) 4"dia. x .226"wall x 24'0"long, 6061T6 mill finish aluminum uprights with double bend 6"offset, welded loop stay connectors and welded aluminum top cap with flag attachment; Crossbars: (four) 4"dia. x .125"wall, 6061T6 mill finish aluminum football crossbars 22'4" and soccer crossbars 23'0" custom extruded with net clip slot; Ground Sleeves: (four) 4.35" o.d. x .100 x 36" 6061T6 aluminum tube with stopbolts at 30" for a “Recessed Installation” or 33" for a “Flush Installation”; Net Stays: (four) 1.688"dia. x .125"wall with a 43"offset and R8"bend; Net Attachment: (one hundred-twenty) MNC Molded Net Clips for attaching net (SN-4/24-W sold separately); Hardware: All stainless steel and galvanized hardware.

  - Model # CAG-NCAA, Uprights: (four) 4"dia. x .125"wall x 18'0"long, 6061T6 mill finish aluminum football uprights with welded aluminum top cap with flag attachment, (four) 4"dia. x .125"wall custom extruded with net clip slot x 10'8"long, 6061T6 mill finish aluminum soccer uprights, welded loop stay connectors; Crossbars: (two) 4"dia. x .125"wall, 6061T6 mill finish fully welded aluminum football and soccer crossbars with two vertical supports spaced equally between crossbars, custom extruded soccer crossbar with net clip slot; Ground Sleeves: (four) 4.35" o.d. x .100 x 40" 6061T6 aluminum tube with stopbolts at 33" for a “Recessed Installation” or 36" for a “Flush Installation”; Net Stays: (four) 1.688"dia. x .125"wall with a 43"offset and R8"bend; Net Attachment: (one hundred-twenty) MNC Molded Net Clips for attaching net (SN-4/24-W sold separately); Hardware: All stainless steel and galvanized hardware.

  - Model # CSF-HS (Square Face-Soccer), Uprights: (four) 4"dia. x .226"wall x 24'0"long, 6061T6 mill finish aluminum uprights with double bend 6"offset with welded square face for soccer, welded loop stay connectors, welded aluminum top cap with flag attachment; Crossbars: (four) 4"dia. x .125"wall , 6061T6 mill finish aluminum football crossbars 22'4" and 4"dia. x .125"wall custom extruded with net clip slot soccer crossbars 23'0" with welded square face; Ground Sleeves: (four) 4.35" o.d. x .100 x 36" 6061T6 aluminum tube with stopbolts at 30" for a “Recessed Installation” or 33" for a “Flush Installation”; Net Stays: (four) 1.688"dia. x .125"wall with a 43"offset and R8"bend; Net Attachment: (one hundred-twenty) MNC Molded Net Clips for attaching net (SN-4/24-W sold separately); Hardware: All stainless steel and galvanized hardware.

  - Model # CSF-NCAA (Square Face-Soccer), Uprights: (four) 4"dia. x .125"wall x 18'0"long, 6061T6 mill finish aluminum football uprights with welded aluminum top cap with flag attachment, (four) 4"sq. x .125"wall custom extruded with net clip slot x 10'8"long, 6061T6 mill finish aluminum soccer uprights, welded loop stay connectors; Crossbars: (two) 4"dia. x .125"wall and 4"sq. x .125"wall custom extruded with net clip slot, 6061T6 mill finish fully welded aluminum football and soccer crossbars with two vertical supports spaced equally between crossbars; Ground Sleeves: (four) 4.35" o.d. x .100 x 40" 6061T6 aluminum tube with stopbolts at 33" for a “Recessed Installation” or 36" for a “Flush Installation”; Net Stays: (four) 1.688"dia. x .125"wall with a 43"offset and R8"bend; Net Attachment: (one hundred-twenty) MNC Molded Net Clips for attaching net (SN-4/24-W sold separately); Hardware: All stainless steel and galvanized hardware.

### Accessories

- Model # CAG/CSF-SG, Soccer Groundbars: 2"sq. x .125" wall custom slotted aluminum side and back groundbar with push-button design for quick & easy removal between soccer and football practices/games, and welded corner receptacles for inter-connecting side & back groundbars. Net Attachment: (one hundred-twenty) MNC Molded Net Clips for attaching net (SN-4/24-W sold separately); Hardware: All stainless steel and galvanized hardware.                                                                                Note: Soccer upright receptacle sleeves for groundbar attachment can be added to Pre-2007 models.Please specify if adding to existing goals.

PART 3	EXECUTION

3.1  		INSPECTION

- Examine the areas and conditions where equipment and systems are to be installed and notify the contractor of conditions detrimental to the proper and timely installation and completion of the work.

- Do not proceed with the work until unsatisfactory conditions have been corrected by the contractor in a manner acceptable and to the satisfaction of the Architect/Engineer or Owner's Representative.

3.2	INSTALLATION

- All athletic equipment shall be installed as indicated on approved submittals as recommended and in strict accordance with manufacturer's written directions and as indicated on the drawings and specified herein.

- All concrete footings for athletic equipment shall be installed as indicated on the drawings and in accordance with Section 03300, Cast-in-Place Concrete.

- All sleeves required for athletic equipment installation shall be set plumb and true to line and grade in concrete as indicated on the drawings and per manufacturer's recommendation.

- All athletic equipment shall be installed in strict accordance with the latest rules, regulations and specifications governing that sport or event for which it is being installed.

3.3  		TESTING, ADJUSTMENT AND OPERATION

- All athletic equipment requiring testing, adjustments and operation shall be tested for proper operation and adjusted to conform to specified standards.

- Provide certifications as required, indicating that equipment has been tested and adjusted to conform to specified standards.

- Provide operating and maintenance instructions and manuals to Owner -designated personnel for the proper operation and care of equipment after equipment has been tested and adjusted to conform to specified standards.

3.4  		CLEANING

- Upon completion of work in any given area, remove all trash and debris from the work area and leave in clean condition.

- END OF SECTION -